# CmdDloader工具使用说明

| 责任人 | 赵亚男 |
| --- | --- |
| 编制部门 | 技术协作部 |
| 发布日期 | 2023-01-04 |

# 版权说明

本文件中出现的任何文字叙述、文档格式、插图、照片、方法、过程等内容，除另有特别注明，均为绝密信息。任何个人、机构未经传音控股的书面授权许可，任何人不得复制、存储、引用或传播本文件的任何片断，无论通过电子形式或非电子形式。

- 背景

- 由于冒烟测试、实验室、工厂自动化测试都需要稳定可靠的自动刷机需求，针对此需求，特此开发此自动化刷机工具；

- 使用方法

Step1:打开cmd命令行输入 CmdDloader.exe -pac 软件包路径

Eg：

- E:\SPRD_CmdDloader202301.04\CmdDloader.exe -pac F:\S662L-F066A-S-OP-20220906V165_user.pac

- Step2:输入完如上指令，使用自动化工具者需要给展锐项目手机发送 adb -s  SN号  reboot autodloader 指令，使手机处于下载模式，然后自动化工具即可自动开始下载。

- 注意：如果需要多台手机进行升级的话，需要打开多个cmd命令行应用程序，按照如上图命令所示发送指令即可

- 多个手机刷机指令发送可以间隔10s

除可实现最简单的刷机以外，还有如下命令可供使用

- "-WriteSN1 <SN1-String>",               “Set with sn1 string; Default is null;" ;

- "-WriteSN2 <SN2-String>",                "Set with sn2 string; Default is null;" ;

- "-MutiFileID <FileID-String>",            "FileID for Multi-project co-software; Default is null;"

- "-MutiProjectID <ProjectID-String>",     "ProjectID for Multi-project co-software; Default is null;"

- "-MutiProjectBase <ProjectBase-String>",  "ProjectBase for Multi-project co-software; Default is null;"

- "-ReadNV <NVFile-String>",       "Read the NV according to the configured NV file; Default is null;" ;

- "-SendFlag",                    "Enable the SendFlag function;";

- "-WriteEfuse",                  "Enable the write fuse function;" ;

- "-WriteTimeStamp",              "Write Time Stamp;" ;

- "-PowerOff",                    "Power off device after download;" （刷机完以后关机）;

- "-Reset",                       "Reset device to normal after download;" （刷机完以后开机）;

- "-EZMode",                   "Only output status and results in the console window;" ;

- "-DDDP",              "Disable detail download process printing; Default is false;" ;

- "-Help",                      "Help information; Default is false;" ;